JFlash Usage Guide

- Preparatice conditions:

- （1）Install JLink with version V7.64e(64 bit) or above in the default install directory，don’t change the install directory；

- After installed, JFlash.exe is located in directory: C:\Program Files\SEGGER\JLink\

- （2）Run install_jflash_env.bat with administrator privileges; If you can’t run this bat script, just copy the files to the specified directory manually;

- Now, you can use JFlash to download firmware to flash.

- Double click JFlash.exe (C:\Program Files\SEGGER\JLink\JFlash.exe)，open JFlash；

- Press “Other...” button，choose the jFlash project file according to the chip you used from directory“...\SEGGER\JLink\Samples\JFlash\ProjectFiles\AdvanChip”；

- After selected the project file, you see the following picture（Here select ACM101LEx.jflash）：

- Make sure JLink is connected with target board and your computer physically, then press “Target--->Connect”:

- After conncected success, the log window displays“Connected successfully”

- Press“File--->Open data file ...”，choose the firmware need to be programmed（hex/bin）:

- Press“Target--->Manual Programming--->Erase Chip”，erase the Flash of chip；

- After erased，the log window display like this：

- Press“Target--->Manual Programming--->Program & Verify” to program firmware；

- After programed, the log window like this：

- Reboot the chip。